Test Execution Report

Device: Moondrop Space Travel 2 (model MD-TWS-046) + Moondrop app

Test Environment

| Device under test | Moondrop Space Travel 2 true wireless earbuds (model MD-TWS-046) |
| --- | --- |
| Companion app | Moondrop (Android) |
| Phone used for testing | Samsung Galaxy A51, One UI 5.1 (Android 13) |

Summary

| Total test cases | 8 |
| --- | --- |
| Passed | 7 |
| Failed | 1 (BUG-001) |
| Pass rate | 7 / 8 = 87.5% |

Results

Full test steps and preconditions are in the accompanying file QA_Moondrop_Space_Travel_2_Test_Cases.xlsx. Status per test case:

| ID | Test title | Status | Notes |
| --- | --- | --- | --- |
| TC-01 | Pairing with the phone | Pass | Paired on the first try, no issues. |
| TC-02 | Auto-reconnect from the case | Pass | Reconnected by itself within a few seconds. |
| TC-03 | Connection range | Fail | Fine in the open room; dropped sooner than expected through a wall. See BUG-001. |
| TC-04 | Play / pause via touch | Pass | Worked as expected. |
| TC-05 | Skip tracks via touch | Pass | Next/previous track both worked. |
| TC-06 | Switching ANC / Transparency mode | Pass | Clear difference between modes, confirmed by voice prompt. |
| TC-07 | Moondrop app detects the earbuds | Pass | Showed as connected, firmware version visible. |
| TC-08 | Applying an EQ preset in the app | Pass | Sound changed noticeably after picking a preset. |

Traceability

Each test case is based on something stated in the official documentation, so it's clear what each one is actually checking:

| What was checked | Where it's described | Test case(s) |
| --- | --- | --- |
| Pairing & reconnecting | Quick start guide | TC-01, TC-02 |
| Connection range | Spec sheet (10 m open-space claim) | TC-03 |
| Touch controls | User manual, gesture table | TC-04, TC-05, TC-06 |
| Moondrop app features | App / Play Store listing | TC-07, TC-08 |

Conclusion

The Moondrop Space Travel 2 and the Moondrop app worked correctly in 7 out of 8 test cases. The one failure was a connection-range issue through a wall (BUG-001), described in the separate bug report. This doesn't affect normal everyday use (listening at home or in the same room as the phone), but it's worth flagging to the developers.